Laboratory Work 3

# Intel x86 CPU instruction set

## Objectives

In this laboratory work, variable declarations and some assembly language instructions are described. First, the general syntax of an instruction is assembly is presented. The exercises in this laboratory work focus on using the presented instructions.

## Variable declaration

In assembly language, a variable is a memory location. By declaring a variable, a specific space is reserved in the memory and a symbolic name is attached to the physical address of the variable. The syntax is the following:

<variable_name>    DB|DW|DD|DQ 	<value1>, [<value2>, <value3>...]

where:

<variable_name> - symbolic name of the variable

DB – Data Byte – 8 bit size

DW – Data Word – 16 bit size or 2 bytes

DD – Data Double – 32 bit size or 4 bytes

DQ – Data Quadruple – 64 bit size or 8 bytes

<valuei> - a constant value

Examples:

var1	db	12h,5, 33, 10101111b

x	dw	1234h, 0ff00h

letter	db	'A'

text 	db	"TEXT"

double	dd	12345678h

Following the above declarations, the memory contains the following:

12   05   21    AF   34   12   00    ff    41    54   45   58    54   78   56    34    12

var1

x

letter

text

double

Another way of reserving a memory block and initializing it with a value or a series of values is the following:

<variable_Name>    DB|DW|DD|DQ  <number> DUP( <value1>[,<value2>..] |?)

where:	<number> - indicates how many locations are reserved

<valuei> - initializing values

? – a zone which is not initializing

Examples:

bloc	db 	100 dup(0)	; 100 bytes of value 0

xx        dw	20 dup(0ffffh)	; 20 memory locations, of size word (16 bits) and initialized with a value of FFFFh

buffer	dd	5 dup(?)	; 5 double words (32 bits each) are reserved

Constants are declared in the same manner as a variable, but some specific rules apply:

<constant_name>		EQU 	<value>

where:	<constant_name> - symbolic name of the constant

EQU – mnemonic for constants ("equals")

<value> - a constant value

Examples:

one	equ	1

true	equ	0ffh

false	equ	0

adr_io	equ	300h		; input/output port address

mask	equ	00100000b	; mask for selection of bit D5

.....

array	db	1,2,3,....

array_length  	equ 	$-tablou	; $-variable is a construction that calculates the size of the memory location

### Assembly language instruction syntax

One instruction occupies one program line and it contains several fields (the brackets indicate that a field can be optional):

[<label>:]	[<mnemonic>	[<parameter_1> [,<parameter_2>]]	[;<comment>]

- <Label> - symbolic name or identifier given in front of an instruction, can have letters, numbers and special characters.

- <Mnemonic> - combination of letters that symbolizes a specific instruction, they are reserved words, cannot be used for other purpose.

- <parameter_1> - the first operand of an instruction and also the destination of the result, can be a register, memory address, variable or an expression that generates a memory address.

- <parameter_2> - the second operand of an instruction, can be anything as described above, in the first parameter, plus it can also be an immediate value/constant.

- <Comment> - ignored by the compiler, a comment symbol is valid for the entire code line.

### Instruction types

#### Transfer instructions

These instructions transfer data between two registers, one register and a memory location or an immediate value is stored in a register or variable. Memory-to-memory transfers are not allowed and both parameters must have the same size.

MOV instruction

[<label>:]	MOV     <parameter_1>, <parameter_2>      [;<comment>]

where:

<parameter_1> = <register>|<reg_segment>|<offset_address>|<variable_name>|<expression>

<parameter_2> = <parameter_1>|<constant>

<register> = EAX|EBX|.....ESP|AX|BX|....SP|AH|AL|....DL

<expression> = [[<index_register>][+<base_register >][+<offset>]]

; the bold brackets are mandatory

<index_register> = SI| DI |ESI | EDI

<base_register> = BX|BP |EBX| EBP

<offset> = <constant>

Examples:

mov ax,bx			et1:	mov ah, [si+100h]

mov cl, 12h				mov al, 'A'

mov dx, var16				mov si, 1234h

mov var32,eax			sf:	mov [si+bx+30h], dx

mov ds, ax				mov bx, cs

Syntax error examples:

mov ax, cl		; different sizes of the operands

mov var1, var2	; both operands are memory locations

mov al, 1234h		; the immediate value/constant is higher than the size of the first operand

LEA instruction

These instructions allow loading in a register a variable address. The first instruction LEA (Load Effective Address) loads in a register (the first parameter) the offset address of the variable from the 2nd parameter.

Examples:

lea  si, var1		; SI<= offset(var1)

lea di, [bx+100]	; DI<= BX+100

XCHG instruction

Exchange the contents of the 2 operands.

XCHG <parameter_1>, <parameter_2>

Note: the second parameter cannot be a constant.

Examples:

xchg al, bh

xchg ax,bx

PUSH and POP instructions

These 2 instructions work by default with the top level of the stack. PUSH adds an operand on the stack, while POP extract the top level value from the stack and stores is in the specified operand. Please note that the stack for x86 works with sizes of 16 and 32 bit.

PUSH <parameter_1>

POP <parameter_1>

Examples:

push ax		push var16

pop bx			pop var16

#### Arithmetic instructions

ADD and ADC instructions

These 2 instructions perform the addition of 2 operands and store the result in the first operand. The ADC instruction also takes into account the carry flag value.

ADD <parameter_1>,<parameter_2>

ADC <parameter_1>,<parameter_2>

Examples:

add ax, 1234h

add bx, ax

adc dx, var16

SUB and SBB instructions

SUB – subtract the second operand from the first, store the result in the first parameter

SBB – subtract with borrow, also subtracts the carry flag value.

SUB    <parameter_1>,<parameter_2>

SBB    <parameter_1>,<parameter_2>

MUL and IMUL instructions

These instructions perform multiplication for unsigned numbers (MUL) and for signed numbers (IMUL). These instructions have only one parameter because, by default, register EAX is used as the first operand (aka. also stores the result).

MUL <parameter_2>

IMUL <parameter_2>

Examples:

mul dh		; AX<=AL*DH

mul bx		; DX:AX<= AX*BX    DX is the extension of AX register

imul var8	; AX<=AL*var8

DIV and IDIV

These instructions perform division for unsigned and signed numbers and, by default, use register EAX for division and result storage.

DIV <parameter_2>

IDIV <parameter_2>

Examples:

div    cl		; AL<=AX/CL and AH<=AX modulo CL (residue of the division)

div    bx	; AX<= (DX:AX)/BX and DX<=(DX:AX) modulo BX

INC and DEC

Perform increase or decrease of the specified parameter (addition or subtraction with a value of one).

INC <parameter>

DEC <parameter>

Examples:

inc   si		; SI<=SI+1

dec  cx		; CX<=CX-1

CMP instruction

Compares the two operands by performing subtraction, but the result is not stored, ony the flags are modified. This instruction is usually followed by a conditional jump instruction.

CMP <parameter_1>, <parameter_2>

Example:

cmp ax, 50h

#### Logical instructions

These instructions implement the basic Boolean algebra, they are performed at bit level and the result is stored in the first operand.

AND, OR, NOT, XOR instructions

AND <parameter_1>,<parameter_2>

OR <parameter_1>, <parameter_2>

NOT <parameter_1>

XOR <parameter_1>,<parameter_2>

Examples:

and   al, 0fh

or     bx, 0000111100001111b

and   al,ch

xor   ax,ax  		; clear register AX

TEST instruction

Logical AND between the 2 operands, but the result is not stored. The purpose is to modify the condition indicators and to avoid the destruction of contents from the first operand.

TEST <parameter_1>,<parameter_2>

Examples:

test al, 00010000b		;checks if bit D4 from AL is zero or not

test bl, 0fh			;checks if the first hexa digit of BL is 0

## Exercises

- Please follow the online document at: http://users.utcluj.ro/~madalin/teaching/SM/labs/W3.pdf