# Laboratory work 7

## Designing Input/Output interfaces

- Design principles

Input/output interfaces adapt the technical-functional characteristics of peripheral devices to the communication rules of a bus. A common interface has 2 parts: one adapted to the signals and operating mode of the connecting bus, and one adapted to the functional peculiarities of the connected device/equipment (e.g. printer, monitor, hard disk, etc.). There are of course standard interfaces, such as: IDE or ATA for storage devices, SVGA/DVI/HDMI for display, MIDI for multimedia, etc.

Figure 1 below exhibits the main block design of an Input/Output interface and it contains the following functional blocks:

- Decoder and selection block

- Signal amplification block

- Input data registers

- Output data registers

- Command and control registers

- Status registers

|  |  |  |  |  |  |  |  |  |  |  |  | Address bus |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Data bus |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Commands bus |  |  |  |  |  |  |  |
|  |  |  |  |  | 2-way amplifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Internal data bus |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Output |  |  | Input |  |  | Command |  |  |  | Status |  |  |  |  |
|  |  | Dec |  |  | registers |  |  | registers |  |  | register |  |  |  | register |  |  |  |  |
|  |  |  |  |  | Output signals |  |  | Input signals |  |  | Command signals |  |  |  | Status signals |  |  |  |  |
|  |  | Figure 1. Main block diagram of a Input/Output interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Decoder and selection block

The purpose of this component is to generate signals to select the interface registers (a.k.a. ports), by decoding the address and command signals from the command bus. Usually, each interface has a base address and a set of relative local addresses. Hence, the address decoding operation has 2 steps: the decoder contains a submodule for decoding the base address (a.k.a. board or interface address) and a second sub-module is responsible for selecting the corresponding registers. Figure 2 below illustrates the decoder components.

|  |  | Address bus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sup. Addr. |  | Board |  |  |  | Inf. Addr. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dec. |  |  |  |  |  | Reg. Addr. |  |  | CS0\ |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Dec. |  |  | CS1\ |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | CS2\ |  |  |  |  |
|  |  | IORC\ |  |  |  | BoardSel |  |  |  |  |  | ….. |  |  |  |  |  |
|  |  | IOWC\ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | CSn\ |  |  |  |  |
|  |  | AEN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Figure 2. Decoder and selection block diagram |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The superior address lines are used to decode the base address, while the inferior addresses for generating the register selection signals (CS0\ … CSn\). The command signals IORC\ and IOWC\ validate the address decoding operation; activating one of these commands indicates the fact that the bus addresses refer to a I/O interface. The AEN signal indicates that the data transfer is done through DMA. If an address is used for both input and output register selection, then the IORC\ and IOWC\ signals validate the selection signals (the dotted line on the above figure). There are 3 types of selecting input and output addresses:

- Total selection – all the address lines are used in decoding addresses.

- Partial selection – only a part of the address lines are used.

- Linear selection – each address line selects a specific register.

Total selection is rarely used because it requires more components for decoding. Instead, all the addresses are available for usage (e.g. 16 addressing lines can be used for 64KB input ports and 64 KB output ports).

Partial selection uses only a part of the addressing lines, hence smaller addressing space. This method is most used because it needs fewer components and most of the times the physical space used by input/output ports does not exceed several hundred bytes. For example, the IBM PC uses only the first 10 address lines, which corresponds to 1KB of addressing space. This space is shared between different interfaces. Some areas were left free for future developments or for user-designed interfaces.

Linear selection is used for dedicated systems with small dimensions (embedded), where the number of input and output ports is very low. In this case, the number of available addresses reduces to the number of addressing lines. The addressing values must be chosen such as a single line must be active (usually logic 0), while the rest are inactive (e.g. FEh=111.1110b, FDh=1111.1101b).

- Amplification block

The number of connected circuits on a bus is limited by the ratio of input and output currents for these circuits. In TTL technology, this ratio is approximately 10, which denotes that 10 circuits can be connected to a bus signal. Hence, the payload of an interface board is limited to a circuit input. In other words, most of the times a data signal must reach several circuits, thus it needs to be amplified. Data and command signals are the most amplified signals; data signals use bidirectional amplifiers, where the direction is selected by a read or write signal, while command signal use common amplifiers.

- Input and output  registers block

These registers assure the data transfer between the connected peripheral device and the system bus. The data transfer depends on the speed of the device, but, usually, is performed on 8 or 16 bits (the interface registers have the same size). Some interfaces allow one-way data transfer (e.g. printer interface), which means that they lack opposite data registers.

Most modern interfaces are built around a specialized circuit, called interface controller. The data, command and status registers are included in these controllers (e.g. serial interface controller, floppy disk interface controller, DMA controller, etc.).

Some recommendations regarding the data registers select signals:

- The select signal for the input registers can be connected to the validation input of the output circuit (CS\ - Chip Select), in order to control the data arriving on the bus (avoid conflict on the data bus).

- The output registers are connected to the registration input of the circuit (CLK - Clock), to assure the data entry in the register; outputs are usually valid all the time.

- Status registers

These registers group a number of signals or bits that indicate the interface and connected device state/condition:

- Operating state of the device (e.g. turned off/on, idle, non-operational, active, etc.)

- Data transfer state: ongoing transfer, temporary stop, data present at input, output register is empty, terminated transfer, etc.

- Error condition: parity error, sync error, missing transfer parameters, etc.

The status information can help the data flow control and can help solve some exception scenarios. Some status signals can be used to generate interrupts.

- Command registers

The command registers allow controlling the interface and the connected device through a program. The control register one can specify:

- The work regime of the interface, especially for the interface controller, which are design for multiple work regimes.

- Data transfer parameters: speed/frequency, number of transferred words, data format, etc.

- Setting or resetting specific interface signals, such as: unit selection, turn on/off, controlling the data flow through specific signals, etc.

2.2. Design example: interface for analog and digital signals acquisition and generation

Analog input/output interfaces are used for connecting analog devices to a PC, common in industry and research laboratories.

Interface design parameters:

- 1 analog output

- 8 analog inputs (multiplexed)

- 4 digital inputs (through optocoupler)

- 4 digital outputs (on relays)

- Base address 300H

- Interface connected to ISA bus

- Interface uses 8 bit data (8 data lines)

The block design of the interface (Figure 3) contains the following components:

- Dec – decoder

- Amp – amplifier

- OR – output register

- I/S R – input and state register

- D/A – digital to analog converter

- A/D – analog to digital converter

- CR  - command register

- Opto – optocoupler

- Relays – for the output digital signals

- Mux – multiplexer for analog signals

|  |  |  |  |  |  |  |  | ISA bus |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Amp |  |  |  |  |  |  |  |  |
|  | Dec |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | OR |  |  | I/S R |  |  |  |  | CR |  |  |  |
|  |  |  |  |  |  |  |  | D/A | A/D |  |  |  |  |  |
|  |  |  | relays |  |  | Opto |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | MUX |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | …. |  |  |  |  |  |
|  |  | Figure 3 Block design of the analog input/output interface |  |  |  |  |  |  |  |  |  |  |  |  |

Address allocation for the interface registers/components:

| Address | Transfer type | Command signal | Register | Select signal |
| --- | --- | --- | --- | --- |
| 300h | Read | IORC\ | Input and status register | CS0R\ |
| 300h | Write | IOWC\ | Output register | CS0W\ |
| 301h | Read | IORC\ | Analog to digital converter | CS1R\ |
| 301h | Write | IOWC\ | Digital to analog converter | CS1W\ |
| 302h | Read | IOWC\ | Command register | CS2\ |

The command register has 3 bits, for the analog channel selection and 1 bit for turning on the analog to digital conversion. The end of the conversion can be detected by reading the I/S register (connection between I/S R and the end of conversion signal), because this register also has the role of the status register.

In Figure 4 the decoder block is illustrated.

|  |  |  | 74LS30 |  |  |  |  |  | 74LS138 |  |  |  | IORC\ |  | IOWC\ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 |  |  |  |  |  | A0 |  | A | O0 |  |  |  |  |  |  |
|  | A8 |  |  |  |  |  | A1 |  | B | O1 |  |  |  |  |  |  |
|  | A7 |  |  |  |  |  | A2 |  | C | O2 |  |  |  |  |  |  |
|  | A6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A5 |  |  |  |  |  |  |  | E1 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | E2\ |  |  |  |  |  |  |  |
|  | IORC\ |  |  |  |  |  |  |  | E3\ |  |  |  |  |  |  |  |
|  | IOWC\ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Figure 4 Decoder block |  |  |  |  |  |  |  |  | CS0R\ CS0W\ CS1R\ CS1W\ CS2\ |  |  |  |  |  |

- Exercises

  - Design a parallel input/output interface for a printer. The table blow contains common signals present in a Centronics type connecter.

| Pin number | Signal name | Direction Printer– PC | Signal description |
| --- | --- | --- | --- |
| 1 | Strobe\ |  | Signal for writing data (valid data) |
| 2-9 | D0-7 |  | Data lines |
| 10 | ACK\ |  | Acknowledge |
| 11 | BUSY\ |  | Printer busy |
| 12 | POUT |  | Paper Out |
| 13 | Sel |  | Printer select |
| 14 | Autofeed\ |  | Automatic generation of LF code |
| 16 | 0V |  | Ground (logic) |
| 17 | CH GND |  | Ground (mechanical) – from the printer case |
| 18 | +5V PULLUP |  | +5V Line terminators |
| 19-29 | GND |  | Ground |
| 31 | RESET\ |  | Reset |
| 32 | FAULT\ |  | Error |
| 36 | SLCTIN\ |  | Select In – sets printer on ON/OFF line |

  - Design an interface for image acquisition from a CCD (Charge Coupled Device) linear camera. A CCD linear camera allows the acquisition of a single line from an image; by moving scenes or objects in the front of the camera (e.g. parts on a conveyor belt) a full image is obtained. The video information of the pixels from a line is generated sequentially on an analog signal (Video), under the control of a clock signal (CLK), generated by the interface. The CCD line is exposed to light as long as signal Exp/Cit\ is active, while the information extraction is done when this signal is not active. The following table contains the signals explained above. Note that a single CCD line has 1024 points/pixels.

| Signal name | Signal type | Direction CCD – PC. | Description |
| --- | --- | --- | --- |
| Video | analog |  | Video signal generated by the camera |
| Exp/Cit\ | digital |  | Exp/Cit\=1 – expose CCD line Exp/Cit\=0 – extract information |
| CLK | digital |  | Clock signal used for extracting video information; each clock period corresponds to 1 pixel in a line |
| GND |  | -- | Ground |
| AGND |  | -- | Analog ground |